Assembly guide

- Parts list

- Subsection assembly

- Final assembly

- Parts list

| - 10 L polystyrene box (x1) |  |
| --- | --- |
| - Electrical box 180mm x 140mm x 80mm (x1) |  |
| - PVC Pipe 1 12" cut to 600mm length (x1) - PVC Pipe 1 12" cut to 180mm length (x1) |  |
| - PVC Coupling 1 12" (x1) - Male adapter 1 12" (x2) - Female adapter 1 12" (x2) |  |
| - PVC reducer bushing 1 12" x 34" (x1) - PVC cap 1 12" (x1) |  |
| - Cable Gland PG13.5 1/2" (x1) - PVC Coupling 3/4" (x1) |  |
| - PVC Pipe 3/4" cut to 50mm length (x1) - PVC Pipe 3/4" cut to 70mm length (x1) - L shape clamp (x2) and screws - Teflon |  |

- Subsection assembly

2.1 Buoy

| - Mark the center of the base and cap of the 10L box. - Drill a 47mm diameter hole for the box base. This corresponds to the diameter of accessory (f). - Drill a 56mm diameter hole for the box cap. This corresponds to the diameter of accessory (e). |  |
| --- | --- |
| - Insert the male adapter (f) into the box base and join them with your preferred elastic sealant. - Insert the coupling (e) to the box cap and join them with your preferred elastic sealant. |  |
| - Inside the box, weld the female adapter (g) with PVC pipe (d) to join the male adapter (f) and coupling (e). |  |
| - Use Teflon on the male adapter (f) thread and adjust the female adapter (g). |  |
| - Finally, weld the bottom section of the remaining male adapter (f) using a small piece of PVC pipe. |  |

2.2 Electronic devices protection

| - Mark the center of the top and bottom sections of the box (b) (140mm x 80mm side). - For the bottom section, drill a 1/2" diameter hole and clean the borders. The cable gland (j) will be placed here. - For the top section, drill a 1/4" diameter hole where the antenna will be located. |  |
| --- | --- |
| - Weld the reducer bushing (h) to the 50mm section of 3/4" PVC pipe (I). |  |
| - Weld the top section of the 3/4" PVC pipe to the coupling (k). |  |
| - Take the thread of element (j) and insert it into the top section of the coupling (k). Use your preferred sealant to join them. |  |
| - Adjust accessory (j) to the box (b) and the thread from the earlier step. |  |
| - Finally, to protect the accessory (j) and increase the box stability, a 70mm section of PVC pipe (m) is added and secured with an L shape clamp and screws (n). |  |

2.3 Sensor probe protection

| - Weld the cap (i) to one side of the PVC pipe (c). |  |
| --- | --- |
| - On the other side, place the female accessory (g). |  |

- Final assembly

| - Take the buoy and probe protection from sections 2.1 and 2.3 and assemble them by screwing the male and female accessories. |  |
| --- | --- |
| - Finally, grab the electronics protection from section 2.2 and adjust the accessory (e) to the accessory (h). |  |